Descriptor (Insert division, branch or project name if required)

[Insert facility name]

Asbestos Management Plan for Leased Facility

[Insert year]

Foreword

This plan has been prepared to communicate how asbestos risks from building materials and plant or equipment, associated with building structures, are managed at this facility leased to the department. This document also serves as a formal record of the Officer in charge delegations established in accordance with the department’s Asbestos management, asbestos incident and work area access permit delegations process.

Site-specific Asbestos Management Arrangements

Facility details

Name: Click or tap here to enter text.

Address: Click or tap here to enter text.

Description of facility use: Click or tap here to enter text.

Officer in charge details

Officer in Charge (OIC): Click or tap here to enter text.

Position title: Click or tap here to enter text.

Phone: Click or tap here to enter text.

Asbestos management training date: Click or tap here to enter text.

Signature:

Plan approval date: Click or tap here to enter text.

Officer in charge delegates

Listed below are persons who have been given a formal delegation of OIC responsibilities for work area access permits under the Work area access permit procedure and Asbestos management, asbestos incident and work area access permit delegations process. Delegates are only be listed if they have completed training as per the Mandatory Annual Training – Ready Reckoner.

|  | Delegate name | Position title | Phone | Date training completed |
| --- | --- | --- | --- | --- |
| 1 | click to insert | click to insert | click to insert | click to insert date |
| 2 | click to insert | click to insert | click to insert | click to insert date |
|  | [add or delete rows as required] |  |  |  |

Trained BEMIR users

Listed below are persons who have completed the Asbestos management training BEMIR learning journey. Note: Training in BEMIR does not qualify a person as an OIC delegate.

|  | Name | Position title | Date training completed |
| --- | --- | --- | --- |
| 1 | click to insert | click to insert | click to insert date |
| 2 | click to insert | click to insert | click to insert date |
|  | [add or delete rows as required] |  |  |

Facility owner details

Facility owner: Click or tap here to enter text.

Phone: Click or tap here to enter text.

Email: Click or tap here to enter text.

Preferred contact method: Click or tap here to enter text.

Lease arrangements

Lease commencement date: Click or tap here to enter text.

Lease end date: Click or tap here to enter text.

Lease content manager reference: Click or tap here to enter text.

Refer to the completed Asbestos in leased facilities checklist – request from person responsible for negotiating or documenting the lease if required

|  | Arrangement | Summary of agreed details (as per lease) | Lease section ref. |
| --- | --- | --- | --- |
| 1 | click to insert | click to insert | click to insert |
| 2 | click to insert | click to insert | click to insert |
| 3 | click to insert | click to insert | click to insert |
| 4 | click to insert | click to insert | click to insert |
|  | [add or delete rows as required] |  |  |

Facility arrangements

If the facility was identified to have asbestos labels placed, nominate the asbestos label management strategy that has been executed:

| ☐ | QBuild has removed all asbestos label (to minimise risk of misrepresentation of asbestos presence at the facility) |
| --- | --- |
| ☐ | QBuild has placed labels on all assumed or confirmed ACM surfaces |
| ☐ | Installation of a revised warning sign to warn against full reliance on the labels placed at the facility |

Departmental asbestos warning signage is installed at the central control point such as reception or office counter.

Date last checked: click to insert date

Asbestos register format in BEMIR:

☐  Facility owner’s asbestos register stored in ‘facility documents’

or

☐  Department-managed asbestos register

Contents

1.	Introduction	6

1.1.	Purpose	6

2.	Asbestos risk profile	7

2.1.	Asbestos in building fabrics	7

2.2.	Asbestos in plant and equipment	7

2.3.	Asbestos in soils	8

3.	Responsibilities	8

4.	Asbestos identification	10

4.1.	Asbestos registers	10

4.1.1.	Asbestos register accessibility	10

4.2.	Asbestos sampling and analysis	11

4.3.	Signage, notices and labelling	11

4.3.1.	Warning signage in facilities	11

4.3.2.	Labelling of ACMs	11

5.	Maintenance of in-situ asbestos	13

6.	Safe work procedures	13

6.1	Planning works	14

6.2	Procuring works	14

6.3	Pre-works	15

6.4	During works	16

6.5	Post works	17

7	Management of asbestos in soils	17

8	Asbestos-related incident management	17

9	Information and training	19

10	Asbestos Management Plan review	19

11	Definitions	20

# Introduction

Asbestos was used extensively by Australian industry, between the 1940s and 1980s, because of its durability, fire resistance and excellent thermal insulating properties.

The manufacture and importation of all asbestos based products was banned in Australia from 31 December 2003 in response to knowledge about the risk of disease from exposure to airborne asbestos fibres.

As with many Queensland buildings, asbestos products were used in the construction of facilities that the department may lease from other entities in order to conduct its operations.

Asbestos risk is regulated under the Work Health and Safety Act 2011 (Qld).  The legislation specifically applies to facilities leased to the department with respect to:

- prohibitions and exceptions for work involving asbestos or asbestos containing material (ACM)

- general duties for managing exposure to airborne asbestos

- requirements for ensuring that certain workplace parties are given access to asbestos registers

- requirements for preparing an asbestos management plan, making it accessible to certain workplace parties (such as workers and service providers) and reviewing and revising the plan.

The Queensland Government Asbestos Management Policy for its Assets (2021) also sets out departmental responsibilities for managing asbestos risk.

In line with the above Policy, the department seeks to avoid entering leases for facilities that contain confirmed or assumed asbestos containing materials (ACM). This is sometimes not possible due to lack of availability of other suitable facilities. Where the leasing of facilities that do not contain asbestos is not possible, the department is committed to the effective management of in-situ ACMs in accordance with arrangements agreed with the facility owners.

## Purpose

The purpose of this Plan is to:

- meet the requirements of section 429 of the Work Health and Safety Regulation 2011

- provide transparency in the asbestos management arrangements in place to safeguard the health of students, departmental staff, service providers and visitors at this site.

# Asbestos risk profile

The asbestos risks at facilities leased to the department arise from a range of factors, including:

- the varying types, ages and uses of buildings;

- the conduct of building, construction and maintenance works at the facilities by service providers, either engaged by the facility owner or the department, or departmental employees;

- key occupants of the facilities – the significant presence of children whose behaviours are unpredictable (by accident or design); and

- limited opportunities to influence asbestos management knowledge of parents/carers and media - impacting perceptions of asbestos risk.

## Asbestos in building fabrics

Common asbestos containing building fabrics that may be present in facilities leased to the department include:

- sheeting and infill panels (internal and external)

- roof sheeting

- ceiling tiles or panels

- vinyl floor coverings

- mastics and fillers, often around doors and windows

- fire doors

- electrical switchboards and meters

- guttering and downpipes

- underground drainage and water pipes

## Asbestos in plant and equipment

Asbestos may be present in equipment and plant that was manufactured and imported up to 31 December 2003, i.e. prior to the manufacture and importation ban.

The types of plant that may contain asbestos include:

- kilns – insulation, gaskets and kiln itself

- mobile plant such as slashers, mowers, tractors, rollers – brakes

- generators and lathes – gaskets

- heater banks - insulation, baffles and lining of pipework and ducts

- heat mats and gloves.

## Asbestos in soils

Materials containing asbestos can become unearthed during work activities such as gardening, landscaping and excavation.  The presence of ACM in soils is usually due to:

- disposal of asbestos containing building materials from historical construction and demolition works at the site

- lack of records of used or unused asbestos pipes and service pits

- illegal disposal of materials on sites prior to their acquisition.

Refer section 7 Management of asbestos in soils.

# Responsibilities

Responsibilities on departmental employees for managing asbestos in facilities leased to the department are set out in the Asbestos management in facilities leased for departmental use procedure. The procedure also references relevant parts of the following procedures:

- Asbestos management procedure

- Asbestos incident management procedure

- Work area access permit procedure.

The procedures can be accessed on the department’s Policy and Procedure Register.

The Officer in charge (OIC) of a facility leased to the department that contains ACM (ACM leased facility) has overall accountability for asbestos management at the facility in terms of the health and safety of the facility community. However, the way in which this accountability is exercised will depend on the agreements specified in the lease, extent of works conducted at the facility and the service delivery models used for the works.

The Asbestos management in facilities leased for departmental use procedure also identifies responsibilities following departmental parties:

- All employees

- Persons responsible for selecting facilities for leasing

- Persons responsible for preparing or negotiating a lease for a facility that contains ACM

- Person responsible for executing a lease for a facility that contains ACM

- Persons responsible for managing a lease for a facility that contains ACM

- Persons responsible for planning work

- Persons responsible for procuring a service provider

- Persons responsible for a service provider contract

- Director, Infrastructure Safety

The responsibilities seek to ensure that appropriate regard is given to:

- the presence of asbestos in facilities prior to leases being established

- asbestos risk while a facility is being used for its usual purpose, for example, educating students

- establishment of agreed arrangements for the maintenance, repair and make-safe of building materials that are assumed or confirmed to contain asbestos.

A person may hold more than one of these responsibilities. For example, if the school principal is involved in selecting a facility for leasing, the principal will hold responsibilities as:

- the person responsible for selecting a facility for leasing

- the OIC.

The OIC may be assisted in carrying out the OIC’s responsibilities for asbestos management at the facility.  However, some of the responsibilities (delegable responsibilities) that relate to the application of the Work area access permit procedure may only be carried out by duly appointed and trained Officer in charge delegates (OIC delegates).  The above ‘Site-specific asbestos management arrangements’ section of this document sets out the formal OIC delegation appointments.

Delegation eligibility and delegable OIC responsibilities are outlined in the Asbestos management, asbestos incident, and work area access permit delegations process.

The asbestos management responsibilities of service providers engaged by the department to deliver work at leased facilities are set out in the Working on Department of Education facilities document.

Departmental employees are assisted in meeting asbestos management responsibilities through, among other things, targeted asbestos management training, access to departmental employees of the Infrastructure Services Division (regional infrastructure, Infrastructure Safety and Disaster management team employees).

# Asbestos identification

The department ensures that asbestos registers are established for facilities leased for departmental use.

Asbestos signage, notices and labels are used to warn persons who may be intending to carry out work on ACM leased facilities about the potential (signage and notices) and actual (labels) presence of asbestos.

## Asbestos registers

The department takes steps to obtain the asbestos register from the facility owner:

- prior to establishment of the lease agreement

- at intervals defined in the lease agreement (for example, annually)

- when the asbestos register is updated.

If the facility owner does not provide a copy of the asbestos register for a building constructed prior to 1 January 1990, the department engages QBuild to:

- conduct an asbestos survey and produce an asbestos register

- monitor the condition of ACM at the facility update the asbestos register accordingly.

The department ensures that all information relating to the condition of asbestos at the ACM leased facility (for example, information relating to damage and deterioration that occurs during the department’s occupancy of the facility) is communicated to the facility owner for the purpose of updating the asbestos register.

### Asbestos register accessibility

The department ensures that an electronic copy of the asbestos register for the ACM leased facility is stored in the Built Environment Materials Information Register (BEMIR) and a printed copy of the asbestos register is made readily available to interested parties (for example, departmental staff, service providers, volunteers) at the facility.

The work area access permit (WAAP) processes incorporate measures that ensure the asbestos register is issued with a WAAP. Asbestos registers are also distributed to departmental service providers during tendering and quoting processes.

## Asbestos sampling and analysis

Laboratory analysis is the only way to confirm the presence of asbestos in a sample of materials. There may be circumstances where the department may elect (with permission of the facility owner) to conduct sampling of building materials in ACM leased facilities that are assumed to contain asbestos, for example, in response to suspected asbestos-related incidents.

All asbestos sampling is conducted by QBuild and areas or rooms subject to sampling are cleared of students and staff and access restricted while sampling works are carried out.

## Signage, notices and labelling

Asbestos labelling and warning signage may be installed at the ACM leased facility by the facility owner. Regardless of the facility owner’s signage, the department ensures warning signage is installed in accordance with the requirements outlined in the Asbestos management procedure, i.e. the placement of at least one publicly visible warning sign at the facility central control point. The requirements outlined in the procedure meet the responsibilities outlined in the Queensland Government Asbestos Management Policy for its Assets – Implementation Standard 1: Minimum Requirements for Asbestos and templated warning signage is compliant with AS1319: Safety signs for the occupational environment.

### Warning signage in facilities

All departmental facilities (except for department-owned housing), which are known or assumed to contain asbestos, have at least one publicly visible warning sign placed at the facility central control point in a location visible to anyone who may disturb ACM (for example, the reception or office counter).  The signage is compliant with AS1319: Safety signs for the occupational environment.

### Labelling of ACMs

Where asbestos labelling is installed at the ACM leased facility, the department ensures that any reported issues with the presence and condition of the labels are managed.

The department has determined that the permanent labelling of all facility ACMs is not reasonably practicable because of the following:

the risk of misrepresentation of asbestos presence to maintenance and construction workers in the event that labels are defaced or removed

the risk of unnecessary alarm to the school community (teachers, parents and carers) by a large number of labels posted across facilities

the difficulties in affixing labels to all potential assumed/confirmed ACM in departmental facilities, particularly window mastics and concealed flooring

significant cost and time for effective implementation

significant cost required to monitor that labels continue to be intact and legible.

The department has identified that a temporary labelling approach (applied for the duration of project works) would:

not likely encounter the above issues

achieve the objective of increased awareness of asbestos presence for workers carrying out works on building materials.

The temporary labelling approach is implemented through the specification of requirements in the Working on Department of Education facilities document, which must be applied by service providers.

The temporary labelling approach requires service providers to:

before commencing the works:

  - prepare a line drawing for the work area that identifies the:

    - surfaces intended to be disturbed during the works

    - presence of assumed/confirmed ACM (based on the content of the asbestos register)

  - affix asbestos labels to the surfaces with assumed/confirmed asbestos that intend to be worked on and ensure it remains for the duration of the works

remove the asbestos labels after the works.

The approach prohibits the service provider from leaving the labels affixed after the works, to prevent any misrepresentation occurring that only labelled building materials at the facility are assumed/confirmed to contain asbestos.

In light of the department’s asbestos label determination, facilities select a label management strategy where an asbestos label approach was implemented prior to the lease being established. The selected strategy is nominated in the ‘Facility arrangements’ section of this plan.

# Maintenance of in-situ asbestos

Lease agreements establish the responsibilities for maintenance of in-situ asbestos at ACM leased facilities. While lease agreements will differ between facilities, the department has processes in place to ensure:

- employees report hazards involving deteriorated ACMs

- appropriate actions are taken in response to such reports (for example restricting access to impacted areas)

- make safe of damaged ACM is conducted by QBuild (where the department is responsible for this action)

- information regarding the condition of ACM at the facility is communicated to the facility owner.

# Safe work procedures

Lease arrangements agreed between the department and a facility owner specify whether the department is responsible for or permitted to undertake any building, construction or maintenance works at the facility. In some cases, the responsibility for all works will remain with the facility owner while some leases may require the department to procure and manage facility-related works. The works may include:

- planned and emergency maintenance activities

- construction works to improve or fit out leased facilities

- use of the facilities for extracurricular activities such as school fetes and sporting events.

The following parties can be involved in these works:

- departmental employees, for example, schools officers undertaking maintenance or minor improvement work such as hanging noticeboards

- service providers who may be engaged to conduct maintenance or project works (such as building extensions, demolitions and refurbishments)

- volunteers, for example, members of Parents and Citizens (P&C) associations undertaking tasks for school fetes.

The department’s asbestos management approach considers asbestos during the lifecycle of works.

## Planning works

The department’s Asbestos management for facilities leased for departmental use procedure requires a consideration of asbestos presence and potential for disturbance during works planned by the department or the facility owner.

Steps being taken to minimise potential impact on the facility community by:

- notifying employees of impending works and potential impacts

- planning to conduct works during a time where students and employees are not present at the facility (for example asbestos removal and other work that may disturb asbestos) – this may require negotiation with the facility owner

- restricting access to work areas

- relocation of departmental operations for the duration of works.

## Procuring works

Where the department is responsible for procurement of works at an ACM leased facility, suitable service providers are selected and engaged in accordance with the department’s Asbestos management procedure.

The procedure requires service providers to be given key asbestos management related documents (including relevant parts of the asbestos register and the Working on Department of Education facilities document) before they price work.  These steps ensure that service providers have information about asbestos risks associated with the work so that they can:

- consider the tools and work methods to be applied for the work – having regard for the legislative prohibitions on the use of certain plant and the limitations on using other plant that may cause the release of airborne asbestos into the atmosphere

- prepare any necessary safe work method statements for the works and ensure their workers are trained in the safe work method statements before commencing the works.

The procedures also ensure that similar outcomes are achieved when the works are managed by a contracted project manager.

Departmental employees responsible for procuring service providers consider the asbestos safety performance history of potential service providers during selection activities.

The department has also established criteria for determining whether service providers are eligible to carry out asbestos removal at departmental facilities.  The criteria leverage industry prequalification arrangements as a means of ensuring that asbestos removal is conducted by service providers with work health and safety maturity.

## Pre-works

Departmental employees involved in planning works take steps to ensure that facilities receive relevant information about works involving assumed or confirmed ACM that may impact the facility community.  This information is communicated to potentially affected parties such as departmental employees, out of hours school care providers, community organisations using the facility and Parents and Citizens’ association members.  These requirements of the Asbestos management procedure support the coordination of necessary asbestos management controls between the facility and service providers.

The department has established work area access permits as a formal authorisation method for granting conditional access to specific work areas in a departmental facility for works conducted by service providers, departmental employees and volunteers.  They provide a vehicle to:

  - allow for planning of departmental operations around works planned by the facility owner (for example finding alternative accommodation for students and staff while work is being completed)

  - communicate departmental requirements such as installation of warning signage and barriers, conduct of risk assessments for electrical hazards, reviewing asbestos registers before commencing works

  - communicate the need for workers to read and comply with the contents of the WAAP

  - record acceptance of departmental conditions for carrying out work

  - identify the departmental representative who has authorised the access

  - identify the service provider who has received the authorisation.

A WAAP is generated from a template (used by multiple government agencies) that is available in BEMIR and must be in place prior to the commencement of works.

The department’s Work Area Access Permit procedure sets out the:

- circumstances for when a WAAP is required for works – generally required for works that have the potential (intentionally or accidentally) to disturb building materials

- processes for creating, issuing and closing WAAPs, including the insertion of specific instructions prohibiting departmental employees and volunteers from undertaking work on assumed or confirmed ACM

- WAAP record management requirements

- WAAP related training requirements

- circumstances of when period WAAPs may be used by departmental employees to create efficiencies at departmental facilities – period WAAPs are not used for work conducted by service providers

- requirements for ensuring a hard copy WAAP template is available at the facility in the event that BEMIR is not accessible.

The signed WAAP is retained at the central control point of the facility for the duration of the works.  The service provider is issued a copy of the facility asbestos register with the WAAP, which must be taken to the work area for review by workers who will be conducting the works.

The Work Area Access Permit procedure also acknowledges the departmental instructions for the conduct of start work meetings before service providers commence works.

## During works

The department has prohibitions in place for a number of activities that may be relevant to ACM leased facilities. These include prohibition of:

- departmental employees and volunteers from carrying out works on assumed or confirmed ACM (including conducting make-safes on assumed or confirmed ACM)

- asbestos removal being conducted during facility school/business hours unless arrangements have been made to ensure facility community members and others who generally use the facility will not be present during the works

- persons other than service providers being at a facility during asbestos removal works unless otherwise approved in accordance with the department’s Asbestos management procedure.

Service providers engaged by the department are bound to comply with WAAPs and the requirements set out in the Working on Department of Education facilities document during the course of their work.  As persons conducting businesses or undertakings, under work health and safety legislation, they are also duty bound to ensure they have safe systems of work for works that will disturb asbestos.  These systems must manage risk to their own workers but also others, such as the facility community, who may be affected by their works.

## Post works

The Work Area Access Permit procedure sets out requirements that the work areas are not to be reoccupied until the work area has been inspected, by the OIC or an OIC delegate, to confirm that there are no visible signs that the area is not safe for reoccupation.

The departmental employee responsible for the service provider contract obtains from the service provider:

- documentation relating to changes to the facility during the works

- supporting certificates (such as asbestos removal clearance and waste disposal certificates, asbestos sampling analysis certificates and ‘clean’ soil certificates).

To support the updating of asbestos register, relevant documents received from the service provider are forwarded to the facility owner or QBuild, depending on who established the asbestos register.

# Management of asbestos in soils

The Working on Department of Education facilities document outlines the steps that must be taken in the event that service providers discover asbestos in soils during the course of works.  These steps involve consultation with the department to confirm the proposed course of action is acceptable.

# Asbestos-related incident management

Asbestos incidents can occur at ACM leased facilities through a range of circumstances, including:

- accidental damage by students, departmental employees and service providers

- intentional damage, for example, damage by vandals

- departmental employees carrying out unauthorised work (departmental employees are prohibited from carrying out works on ACM)

- service providers carrying out work on ACM without the necessary controls

- service providers carrying out work on ACM that, due to the risk to the workers and other persons, is prohibited under work health and safety legislation

- service providers leaving suspected ACM dust, debris or loose ACM after works

- weather events.

The department’s Asbestos incident management procedure provides directions to departmental employees for incidents that occur during:

- general facility business operations, i.e., the activities associated with the primary purpose of the facility

- works undertaken by service providers, where the incident affects or potentially affects the safety of the facility community:

    - during the facility's business operations, for example, potential exposure to asbestos created as a result of a service provider accidentally damaging an asbestos containing school hallway wall during school operations, or

    - when facility business operations recommence, for example, weekend works leaving assumed or confirmed ACM materials with unsealed penetrations.

The Working on Department of Education facilities document sets out the steps to be taken by a service provider if they have had an asbestos incident that affects or potentially affects the safety of the facility community.

The Asbestos management in facilities leased for departmental use procedure and relevant sections of the Asbestos incident management procedure set out steps for:

- initiating an immediate response, which includes (for a facility that is a school) stopping work, restricting access, installing warning signage and reporting the incident

- securing the scene of a suspected asbestos dangerous incident (until a decision is made that the incident is not notifiable to the Work Health and Safety Regulator or if notifiable the Regulator gives permission for the scene to be disturbed)

- checking the asbestos register to assist determinations about whether the incident does involve assumed or confirmed ACM

- contacting the facility owner or QBuild to initiate make-safe actions, sample assumed ACM involved in the incident and disposal of materials

- managing clothing that has come into contact with dust suspected to contain asbestos

- communicating incident details to relevant employees

- recording asbestos-related incidents

- notifying parents/carers where students have been in proximity of an incident

- supporting departmental employees and parents/carers who have concerns associated with an incident

- the investigation of particular incidents, such as those that are notifiable to the Work Health and Safety Regulator and those where children were in an enclosed space within 10 minutes of a power tool having been used on ACM or assumed ACM in that enclosed space

- managing asbestos-related incident records.

As persons conducting businesses or undertakings, service providers are duty bound by work health and safety legislation to ensure they take appropriate steps to protect the safety of themselves, their workers and others and manage incidents accordingly.

# Information and training

The department has initiated multiple information and training measures to ensure persons under the management and control of facilities are aware of the risks from the presence of asbestos in the workplace and actions to be taken to protect their health.  These measures include:

- mandatory annual training of departmental employees that incorporates key prohibitions and safety precautions associated with asbestos risk

- induction training for cleaners and schools officers (as the employees most likely to discover and interact with building materials respectively)

- mandatory five yearly training of OICs and OIC delegates on asbestos risk and responsibilities under the asbestos related departmental procedures

mandatory training of others with responsibilities under the department’s procedures relating to asbestos management

- mandatory training for employees assisting OICs through the use of BEMIR

- dedicated asbestos management intranet webpage with resources to support persons with responsibilities under departmental procedures relating to asbestos management.

The department has imposed minimum awareness training that service providers and their workers must have if intending to carry out work on departmental facilities.  The requirements are outlined in the Working on Department of Education facilities document.

# Asbestos Management Plan review

The responsibilities for the preparation and review of site specific asbestos management plans for ACM leased facilities are set out in the department’s Asbestos management in facilities leased for departmental use procedure.  The procedure includes details of the frequencies for the updating of Leased facility asbestos management plans and the management of superseded versions of plans.  The date of the last update of a leased facility asbestos management plan is identified in the Officer in charge details section of the plan.  The department’s Leased facility asbestos management plan template is published on the department’s Policy and Procedure Register.  The register identifies the version number and version effective date of the template.

# Definitions

| Term | Definition |
| --- | --- |
| Facility community member | Persons who are generally present at a facility, for example, at a school the facility community would include teaching staff, administration staff, students and visiting parents and carers |
| Officer in charge | The person identified in departmental procedures to have accountability for managing asbestos at a facility. |
| Officer in charge delegate | A departmental employee who has received formal delegation from the Officer in Charge of a facility to perform delegable responsibilities for the Officer in Charge. |
| Periodic inspections | Inspections conducted periodically to determine whether assumed or confirmed ACM has changed condition. |
| Plant | As defined under the Work Health and Safety Act, i.e. includes - any machinery, equipment, appliance, container, implement and tools; - any component of any of those things; and - anything fitted or connected to any of those things. |